Pin in/out TL1000R ECU 32920-02FAO, 112100-0290 12V NEP021

ECU pin Bike Loom Connection Board label CPU pin-label

34 Pin Plug

1 Ignition Coil 1 IG1 5 CTS10A

2 Ignition Coil 2 IG2 6 CTS10B

- Ignition Coil Fault 10 CTD5

3 Not Connected _ No Circuit

4 Not Connected _ No Circuit

5 Front (1) Primary injector #11 1 CTS14A

- Front (1) Primary injector Fault 128 CTS16A

6 Rear (2) Primary injector #11 129 CTS14B

- Rear (2) Primary injector Fault 127 CTS16B

7 Tachometer signal TAC 86 SIZ0/PE6

8 Vacuum Solenoid Valve SOL 102 DATA8/PG0

9 Fuel Pump Relay FP 87 AS/PE5

10 Ground connection E01 -

11 Not Connected _ No Circuit

12 Not Connected _ No Circuit

13 Not Connected Comp’s not installed SOL1 101 DATA9/PG1

14 Front (1) Secondary injector #12 7 CTS8A

- Front (1) Secondary injector Fault 43 ADDR14/PA3

15 Rear (2) Secondary injector #22 6 CTS8B

- Rear (2) Secondary injector Fault 44 ADDR15/PA4

16 Clutch Lever CLT 31 ADDR3/PB0

17 +12v +B -

18 Ground connection E02 -

19 Not Connected _ No Circuit

20 Not Connected _ No Circuit

21 Not Connected _ No Circuit

22 Not Connected Comp’s not installed SSW No Circuit

23 Start input STA 33 ADDR5/PB2

24 Dealer Mode Select TS 35 ADDR6/PB3

25 External ground (Map AB select?) MS 36 ADDR7?PB4

26 Ground connection Coil 1&2 E03 -

27 Not Connected _ No Circuit

28 Not Connected _ No Circuit

29 Not Connected Vpp VFP

30 Not Connected FCCP 88 DS/PE4

31 Not Connected ST 52 TXDA/PMC7 &

53 RXDA/PMC6

32 Not Connected _ No Circuit

33 Not Connected _ No Circuit

34 Neutral NT 37 ADDR8/PB5

26 Pin Plug

35 Ground Connection E1 -

36 Crank Position Sensor N+ 8 CTS6B

37 Camshaft Position Sensor G+ 9 CTS6A

38 Injector Voltage Compensation VM 20 AN4/PADA4

39 Throttle Position Sensor VTA 24 AN0/PADA0

40 Intake Air Pressure PM 23 AN1/PADA1

41 VCC VCC _

42 Not Connected _ No Circuit

43 Crank Position Sensor N- 8 CTS6B

44 Camshaft Position Sensor G- 9 CTS6A

45 Intake Air Temp THA 21 AN3/PADA3

46 Engine Coolant Temp THW See IC301

47 Ambient Air Pressure PA See IC301

48 Not Connected Comp’s not installed CS3 81 IRQ2/PF2

49 Yoshi Box Connector CS1 38 ADDR9/PB6

50 Yoshi Box Connector CS2 39 ADDR10/PB7

51 Not Connected Comp’s not installed KNK 32 ADDR4/PB1

52 To Instrument Cluster TECH 54 TXDB/PMC5 &

55 RXDB/PMC4

53 Gear Position GP 19 AN5/PADA5

54 Ground Connection E2 -

55 Yoshi Box Connector COV1 See IC301

56 Yoshi Box Connector COV2 See IC301

57 Yoshi Box Connector COV3 22 AN2/PADA2

58 Not Connected Comp’s not installed COV4 See IC301

59 Tip over Sensor DON See IC301

60 Ignition Switch (Immobilizer?) AT See IC301

IC301 Pin in/out

IC301 is a Toshiba 4052BF

Pin Connection

1 ECU 46 THW Engine Coolant Temp

2 ECU 47 PA Ambient Air Pressure

3 CPU 15 AN7/PADA7

4 ECU 59 DON Tip over Sensor

5 ECU 60 AT Ignition Switch

9 CPU 89 PE3

10 CPU 90 AVEC/PE2

11 ECU 59 COV4 Not Connected

12 No circuit

13 CPU 16 AN6/PADA6

14 ECU 55 COV1 Yoshi Box

15 ECU 56 COV2 Yoshi Box

IC502 (ATMEL 93C66) Connection Details

Pin 1 CS to CPU pin 103 DATA7/PH7

Pin 2 SK to CPU pin 51 SCK/PCM2

Pin 3 DI to CPU pin 50 MOSI/PMC1

Pin 4 DO to CPU pin 49 MISO/PMC0

Pin 5 GR to Ground

Pin 6 ORG to Ground

Pin 7 DC   Not connected

Pin 8 VCC to VCC

Code Sections

Programme initialization starts at 1000h

Main Loop is 12A4h -> 1376h

Subr 24A0 called if PE

RAM Memory MAP

000E	PORTB shadow register.

0203	AN6/AN7 multiplexer select

212	Yosh Box value

214	Yosh Box value

216	Yosh box value

218	Yosh box value